Casino

Guide Specification

This Painting Schedule is furnished as a guide

to select paint systems, and is not all-inclusive of

available Sherwin-Williams products. Specifiers

should review the contents and edit to suit the

particular project and its location.

The schedule is arranged by substrates, and

offers latex, acrylic, water-based epoxy, and

water-based urethane. Each type also includes

the available sheens.

Local and National V.O.C. (Volatile Organic

Compound) regulations have been taken into

consideration.  However, because regulations vary

by region and frequently change, specifiers should

verify that the products comply with local rules.

Consult with your Sherwin-Williams representative

before finalizing the selection.

If you need additional product information, contact

your Sherwin-Williams representative, refer to the

Sherwin-Williams Painting Systems Catalog, visit

sherwin-williams.com or call our Architectural

Services Answer Team.

The Sherwin-Williams Company

Architectural Services Department

800-321-8194 (Telephone)

216-566-1392 (Fax)

specifications@sherwin.com (Email)

09 91 23 INTERIOR &

09 91 13 EXTERIOR

Issued July 2015

The Sherwin-Williams Company

Download this document at

www.swspecs.com

— Page 1

Table of Contents

INTERIOR SYSTEMS:

Important Surface Prep Instructions:	Page 3

Interior Topcoats:	Page 3

Miscellaneous Notes:	Page 4

Guest Room:	Page 5

Gypsum Board — Walls

Gypsum Board — Ceiling

Ferrous Metal — Doors, Frames, and Miscellaneous Metals

Wood — Painted Doors, Frames, Trim, or Chair Rails

Wood — Stained Trim or Chair Rails

Hallway/Corridor:	Page 6

Gypsum Board — Walls

Gypsum Board — Ceiling

Ferrous Metal — Doors, Frames, and Miscellaneous Metals

Ferrous Metal — High Performance Finish (including Handrails)

Non-Ferrous Metal — Galvanized/Aluminum Surfaces

Non-Ferrous Metal — High Performance Finish (including Handrails)

Lobby/Common Area: 	Page 7

Gypsum Board — Walls

Gypsum Board — Ceiling

Ferrous Metal — Doors, Frames, and Miscellaneous Metals

Ferrous Metal — High Performance Finish (including Handrails)

Non-Ferrous Metal — Galvanized/Aluminum Surfaces

Non-Ferrous Metal — High Performance Finish (including Handrails)

Conference/Multi-Purpose Room:	Page 8

Gypsum Board — Walls

Gypsum Board — Ceiling

Ferrous Metal — Doors, Frames, and Miscellaneous Metals

Bathroom:	Page 8 – 9

Gypsum Board — Walls

Gypsum Board — Ceiling

Ferrous Metal — Doors, Frames, and Miscellaneous Metals

Fitness Center:	Page 9

Gypsum Board — Walls

Gypsum Board — Ceiling

Ferrous Metal — Doors, Frames, and Miscellaneous Metals

Restaurant/Kitchen:	Page 10

Gypsum Board — Walls

Gypsum Board — Ceiling

Ferrous Metal — High Performance Finish (including Handrails)

Non-Ferrous Metal — High Performance Finish (including Handrails)

Non-Ferrous Metal — Galvanized/Aluminum Surfaces

Page 2 —

Table of Contents (Cont.)

Bar:	Page 11

Gypsum Board — Walls

Gypsum Board — Ceiling

Ferrous Metal — High Performance Finish (including Handrails)

Non-Ferrous Metal — Galvanized/Aluminum Surfaces

Non-Ferrous Metal — High Performance Finish (including Handrails)

Nightclub:	Page 12

Gypsum Board — Walls

Gypsum Board — Ceiling

Ferrous Metal — Doors, Frames, and Miscellaneous Metals

Casino/Gaming: 	Page 13

Gypsum Board — Walls

Gypsum Board — Ceiling

Ferrous Metal — Doors, Frames, and Miscellaneous Metals

Ferrous Metal — High Performance Finish (including Handrails)

Non-Ferrous Metal — Galvanized/Aluminum Surfaces

Non-Ferrous Metal — High Performance Finish (including Handrails)

BOH Areas:	Page 14 – 15

Gypsum Board — Walls

Gypsum Board — Ceilings

Warehouse/Distribution

Boiler Room/Physical Plant/Maintenance Office

Indoor/Outdoor Pool:	Page 16

Pool Deck — Waterproofing Deck Coating

CMU — Interior Walls in Pool Area & Chemical Storage

Gypsum Board — High Performance Finish for Walls & Ceilings

Faux Impressions — Decorative Wall Finishes; Metallic and High-Polish Fresco (Venetian Plaster):	Page 16

Concrete Floors — Pigmented Coatings Systems:	Page 17

Light Duty (Foot Traffic)

Medium/Heavy Duty (Food Service, Restroom (non-Shower), Laundry, Corridor)

Concrete Floors — Decorative Coatings Systems:	Page 17

EXTERIOR SYSTEMS:

Exterior Topcoats:	Page 18

Concrete & Masonry Vertical Surfaces — Smooth:	Page 19

Plaster/Stucco/EIFS:	Page 19

Wood:	Page 20

Ferrous Metals:	Page 20

Non-Ferrous Metals:	Page 20

Balcony:	Page 20

Casino Facility Interior Paint Schedule

Important Surface Preparation Instructions:

Verify the existence of lead based paints on the project. Buildings constructed after 1978 are less likely to contain lead based paints. If lead based paints are suspected on the project, all removal must be done in accordance with the EPA Renovation, Repair and Painting rule and all applicable state and local regulations. State and local regulations may be more strict than those set under the federal regulations. Verify that Owner has completed a Hazardous Material Assessment Report for the project prior to issuing of Drawings.

WARNING! Removal of old paint by sanding, scraping or other means may generate dust or fumes
that contain lead. Exposure to lead dust or fumes may cause brain damage or other adverse health
effects, especially in children or pregnant women. Controlling exposure to lead or other hazardous
substances requires the use of proper protective equipment, such as a properly fitted respirator
(NIOSH approved) and proper containment and cleanup. For more information, call the National 
Lead Information Center at 1-800-424-LEAD (in US) or contact your local health authority. Removal
must be done in accordance with EPA Renovation, Repair and Painting Rule and all related state
and local regulations. Care should be taken to follow all state and local regulations which may be
more strict than those under the federal RRP Rule.

Specifier Notes — Topcoats:

ProMar® 400 Zero VOC Interior Latex — A durable, professional quality zero VOC vinyl acrylic topcoat available in a Flat, Low Sheen, Eg-Shel, and Semi-Gloss finishes.

b. ProMar® 200 Zero VOC Interior Latex — A durable, professional quality zero VOC vinyl acrylic topcoat available in a Flat, Low Sheen, Eg-Shel, and Semi-Gloss finishes.

c. Pro Industrial™ Pre-Catalyzed Waterbased Epoxy — A single-component, waterbased acrylic epoxy that offers the durability and resistance to stains and most cleaning solvents usually characteristic of two-component waterbased epoxies. Available in Eg-Shel and Semi-Gloss finishes.

d. Pro Industrial™ Water Based Catalyzed Epoxy — A two-component water based epoxy formulated to provide excellent corrosion resistance, chemical resistance, abrasion resistance, and moisture resistance. Available in Eg-Shel and Gloss finishes.

e. Pro Industrial™ Acrylic Coating — A single-component, light industrial, 100% acrylic coating that flows and levels to a smooth finish. Available in Eg-Shel, Semi-Gloss, and Gloss finishes.

f. Faux Impressions® Specialty Finishes — Water-based, these unique finishes feature subtle details not found with standard paint in order to maximize visual interest, create a dramatic environment and highlight special areas or focal points within the space.

Recommended Finishes:

Ceilings and Soffits:	Flat Finish

Walls:	Eg-Shel or Semi-Gloss Finish

Trim:	Semi-Gloss Finish (unless otherwise noted)

— Page 3

Recommended Finishes:

Ceilings and Soffits:	Flat, Eg-Shel or Semi-Gloss Finish

Walls:			Eg-Shel or Semi-Gloss Finish

Trim:			Semi-Gloss Finish (unless otherwise noted)

Miscellaneous Notes:

a.	For previously painted substrates (when clean, dry and dull), the primer can often be eliminated and then

finished with 1 or 2 coats. When loose or peeling paint is evident or surfaces have a gloss finish, contact your

Sherwin-Williams Architectural Account Executive for specific preparation and finishing instructions.

b.	This schedule is not inclusive of all SW products; other products and finishes are available.

c.	All products listed meet LEED-09 New Construction, LEED-09 Healthcare and SCAQMD requirements, unless

otherwise noted.

d.	For questions and assistance (including MPI standards ), please contact your local Architectural Account Executive.

e.	Some colors may not be zero VOC after tinting with conventional colorants. Refer to the current MSDS/EDS for

exact VOCs. VOCs may vary by base.

Guest Room:

Gypsum Board — Walls

Eg-Shel Finish — Low Odor Zero VOC System

Primer:	ProMar 200 Zero VOC Interior Latex Primer, B28W2600, 0 g/L VOC

Or Harmony Interior Latex Primer, B11W500 — Odor Eliminating Technology, 0 g/L VOC

1st coat:	ProMar 200 Zero VOC Interior Latex Eg-Shel, B20W2650 series, 0 g/L VOC

Or Harmony Interior Latex Eg-Shel, B9W551 — Odor Eliminating Technology, 0 g/L VOC

2nd coat:	ProMar 200 Zero VOC Interior Latex Eg-Shel, B20W2650 series, 0 g/L VOC

Or Harmony Interior Latex Eg-Shel, B9W551 — Odor Eliminating Technology, 0 g/L VOC

Semi-Gloss Finish: Low Odor Zero VOC System

Primer:	ProMar 200 Zero VOC Interior Latex Primer, B28W2600, 0 g/L VOC

Or Harmony Interior Latex Primer, B11W500 — Odor Eliminating Technology, 0 g/L VOC

1st coat:	ProMar 200 Zero VOC Interior Latex Semi-Gloss, B31W2651 series, 0 g/L VOC

Or Harmony Interior Latex Semi-Gloss, B10W551 — Odor Eliminating Technology, 0 g/L VOC

2nd coat:	ProMar 200 Zero VOC Interior Latex Semi-Gloss, B31W2651 series, 0 g/L VOC

Or Harmony Interior Latex Semi-Gloss, B10W551 — Odor Eliminating Technology, 0 g/L VOC

Gypsum Board — Ceiling

Flat Finish — Low Odor Zero VOC System

Primer:	ProMar 200 Zero VOC Interior Latex Primer, B28W2600, 0 g/L VOC

Or Harmony Interior Latex Primer, B11W500 — Odor Eliminating Technology, 0 g/L VOC

1st coat:	ProMar 200 Zero VOC Flat, B30W2650 series, 0 g/L VOC

Or Harmony Interior Latex Flat, B5W551 series — Odor Eliminating Technology, 0 g/L VOC

2nd coat:	ProMar 200 Zero VOC Flat, B30W2650 series, 0 g/L VOC

Or Harmony Interior Latex Flat, B5W551 series — Odor Eliminating Technology, 0 g/L VOC

Page 4 —

Casino Facility Interior Paint Schedule

Miscellaneous Notes:

SURFACE PREPARATION:

A.	Proper product selection, surface preparation, and application affect coating performance. Coating integrity and service life will be reduced because of improperly prepared surfaces. Selection and implementation of proper surface preparation ensures coating adhesion to the substrate and prolongs the service life of the coating system.

B.	Selection of the proper method of surface preparation depends on the substrate, the environment, and the expected service life of the coating system. Economics, surface contamination, and the effect on the substrate will also influence the selection of surface preparation methods.

C.	The surface must be dry and in sound condition. Remove oil, dust, dirt, loose rust, peeling paint or other contamination to ensure good adhesion.

- Remove mildew before painting by washing with a solution of 1 part liquid household bleach and 3 parts of warm water. Apply the solution and scrub the mildewed area. Allow the solution to remain on the surface for 10 minutes. Rinse thoroughly with clean water and allow the surface to dry 48 hours before painting. Wear protective glasses or goggles, waterproof gloves, and protective clothing. Quickly wash off any of the mixture that comes in contact with your skin. Do not add detergents or ammonia to the bleach/water solution.

- Previously Coated Surfaces: maintenance painting will frequently not permit or require complete removal of all old coatings prior to repainting. However, all surface contamination such as oil, grease, loose paint, mill scale, dirt, foreign matter, rust, mold, mildew, mortar, efflorescence, and sealers must be removed to assure sound bonding to the tightly adhering old paint. Glossy surfaces of old paint films must be clean and dull before repainting. Thorough washing with an abrasive cleanser will clean and dull in one operation, or, wash thoroughly and dull by sanding. Spot prime any bare areas with an appropriate primer. Recognize that any surface preparation short of total removal of the old coating may compromise the service length of the system. Check for compatibility by applying a test patch of the recommended coating system, covering at least 2 to 3 square feet. Allow to dry one week before testing adhesion per ASTM D3359. If the coating system is incompatible, complete removal is required per ASTM D4259.

- No exterior painting should be done immediately after a rain, during foggy weather, when rain is predicted, or when the temperature is below 50°F, unless products are designed specifically for these conditions.  On large expanses of metal siding, the air, surface and material temperatures must be 50°F or higher to use low temperature products.

- This schedule is not inclusive of all Sherwin-Williams products; other products and finishes are available.

H.	Not all products are compliant nationwide, visit related websites for CARB, OTC, SCAQMD for current VOC regulations.

CARB — http://www.arb.ca.gov/coatings/coatingsrules.htm

OTC — www.otcair.org

SCAQMD — http://www.aqmd.gov/rules

I.	Product names, logos, brands and other trademarks are the property of their respective trademark holders.

J. 	Refer to the current MSDS/EDS for specific VOCs (calculated per 40 CFR 59.406). VOCs may vary by base and sheen.

K.	Coating Maintenance Manual: upon conclusion of the project, the Contractor or paint manufacture/supplier shall furnish a coating maintenance manual, such as Sherwin-Williams “Custodian Project Color and Product Information” report or equal. Manual shall include an Area Summary with finish schedule, Area Detail designating where each product/color/finish was used, product data pages, Material Safety Data Sheets, care and cleaning instructions, touch-up procedures, and color samples of each color and finish used.

Guest Room:

Gypsum Board — Walls

Semi-Gloss Finish

Primer:	ProMar® 200 Zero VOC Interior Latex Primer, B28W2600

1st coat:	ProMar® 200 Zero VOC Interior Latex Semi-Gloss, B31-2600 Series

2nd coat:	ProMar® 200 Zero VOC Interior Latex Semi-Gloss, B31-2600 Series

Semi-Gloss — High Performance Finish

Primer:	ProMar® 200 Zero VOC Interior Latex Primer,  B28W2600

1st coat:	Pro Industrial™ Pre-Catalyzed Water Based Epoxy Semi-Gloss, K46 Series

2nd coat:	Pro Industrial™ Pre-Catalyzed Water Based Epoxy Semi-Gloss, K46 Series

Eg-Shel Finish

Primer:	ProMar® 200 Zero VOC Interior Latex Primer, B28W2600

1st coat:	ProMar® 200 Zero VOC Interior Latex Eg-Shel, B20-2600 Series

2nd coat:	ProMar® 200 Zero VOC Interior Latex Eg-Shel, B20-2600 Series

Eg-Shel — High Performance Finish

Primer:	ProMar® 200 Zero VOC Interior Latex Primer, B28W2600

1st coat:	Pro Industrial™ Pre-Catalyzed Water Based Epoxy Eg-Shel, K45 Series

2nd coat:	Pro Industrial™ Pre-Catalyzed Water Based Epoxy Eg-Shel, K45 Series

Gypsum Board — Ceiling

Flat Finish

Primer:	ProMar® 200 Zero VOC Interior Latex Primer, B28W2600

1st coat:	ProMar® 400 Zero VOC Interior Latex Flat, B30-4600 Series

2nd coat:	ProMar® 400 Zero VOC Interior Latex Flat, B30-4600 Series

Ferrous Metal — Doors, Frames, and Miscellaneous Metals

Semi-Gloss Finish

Primer:	Pro Industrial™ Pro-Cryl® Universal Primer, B66-310 Series

1st coat:	Pro Industrial™ Acrylic Semi-Gloss, B66-650 Series

2nd coat:	Pro Industrial™ Acrylic Semi-Gloss, B66-650 Series

Eg-Shel or Gloss — High Performance Finish

Primer:	Pro Industrial™ Pro-Cryl® Universal Primer, B66-310 Series

1st coat:	Pro Industrial™ Water Based Catalyzed Epoxy Eg-Shel, B73-360 Series or Gloss, B73-300 Series

2nd coat:	Pro Industrial™ Water Based Catalyzed Epoxy Eg-Shel, B73-360 Series or Gloss, B73-300 Series

Wood — Painted Doors, Frames, Trim, or Chair Rails

Semi-Gloss Finish

Primer:	PrepRite® ProBlock® Interior Latex Primer Sealer B51 Series

1st coat:	Pro Industrial™ Acrylic Semi-Gloss, B66-650 Series

2nd coat:		Pro Industrial™ Acrylic Semi-Gloss, B66-650 Series

Wood — Stained Trim or Chair Rails

Stained Finish

Stain:	Minwax® Wood 250 Stain

2nd coat:	Minwax® Water based Polyurethane Varnish, Satin or Gloss

3rd coat:	Minwax® Water based Polyurethane Varnish, Satin or Gloss

— Page 5

— Page 3

Hallway/Corridor:

Gypsum Board — Walls

Semi-Gloss Finish

Primer:	ProMar® 200 Zero VOC Interior Latex Primer, B28W2600

1st coat:	ProMar® 200 Zero VOC Interior Latex Semi-Gloss, B31-2600 Series

2nd coat:	ProMar® 200 Zero VOC Interior Latex Semi-Gloss, B31-2600 Series

Semi-Gloss — High Performance Finish

Primer:	ProMar® 200 Zero VOC Interior Latex Primer, B28W2600

1st coat:	Pro Industrial™ Pre-Catalyzed Water Based Epoxy Semi-Gloss, K46 Series

2nd coat:	Pro Industrial™ Pre-Catalyzed Water Based Epoxy Semi-Gloss, K46 Series

Eg-Shel Finish

Primer:	ProMar® 200 Zero VOC Interior Latex Primer, B28W2600

1st coat:	ProMar® 200 Zero VOC Interior Latex Eg-Shel, B20-2600 Series

2nd coat:	ProMar® 200 Zero VOC Interior Latex Eg-Shel, B20-2600 Series

Eg-Shel — High Performance Finish

Primer:	ProMar® 200 Zero VOC Interior Latex Primer, B28W2600

1st coat:	Pro Industrial™ Pre-Catalyzed Water based Epoxy Eg-Shel, K45 Series

2nd coat:	Pro Industrial™ Pre-Catalyzed Water Based Epoxy Eg-Shel, K45 Series

Gypsum Board — Ceiling

Flat Finish

Primer:	ProMar® 200 Zero VOC Interior Latex Primer, B28W2600

1st coat:	ProMar® 400 Zero VOC Interior Latex Flat, B30-4600 Series

2nd coat:	ProMar® 400 Zero VOC Interior Latex Flat, B30-4600 Series

Ferrous Metal — Doors, Frames, and Miscellaneous Metals

Semi-Gloss Finish

Primer:	Pro Industrial™ Pro-Cryl® Universal Primer, B66-310 Series

1st coat:	Pro Industrial™ Acrylic Semi-Gloss, B66-650 Series

2nd coat:	Pro Industrial™ Acrylic Semi-Gloss, B66-650 Series

Ferrous Metal — High Performance Finish (Including Handrails)

Eg-Shel or Gloss Finish

Primer:	Pro Industrial™ Pro-Cryl® Universal Primer, B66-310 Series

1st coat:	Pro Industrial™ Water Based Catalyzed Epoxy Eg-Shel, B73-360 Series or Gloss, B73-300 Series

2nd coat:	Pro Industrial™ Water Based Catalyzed Epoxy Eg-Shel, B73-360 Series or Gloss, B73-300 Series

Non-Ferrous Metal — Galvanized/Aluminum Surfaces

Semi-Gloss Finish

Primer:	Pro Industrial™ Pro-Cryl® Universal Primer, B66-310 Series

1st coat:	Pro Industrial™ Acrylic Semi-Gloss, B66-650 Series

2nd coat:	Pro Industrial™ Acrylic Semi-Gloss, B66-650 Series

Non-Ferrous Metal — High Performance Finish (Including Handrails)

Eg-Shel or Gloss Finish

Primer:	Pro Industrial™ Pro-Cryl® Universal Primer, B66-310 Series

1st coat:	Pro Industrial™ Water Based Catalyzed Epoxy Eg-Shel, B73-360 Series or Gloss, B73-300 Series

2nd coat:	Pro Industrial™ Water Based Catalyzed Epoxy Eg-Shel, B73-360 Series or Gloss, B73-300 Series

Page 6 —

— Page 7

Lobby/Common Area:

Gypsum Board — Walls

Semi-Gloss Finish

Primer:	ProMar® 200 Zero VOC Interior Latex Primer, B28W2600

1st coat:	ProMar® 200 Zero VOC Interior Latex Semi-Gloss, B31-2600 Series

2nd coat:	ProMar® 200 Zero VOC Interior Latex Semi-Gloss, B31-2600 Series

Semi-Gloss — High Performance Finish

Primer:	ProMar® 200 Zero VOC Interior Latex Primer, B28W2600

1st coat:	Pro Industrial™ Pre-Catalyzed Water Based Epoxy Semi-Gloss, K46 Series

2nd coat:	Pro Industrial™ Pre-Catalyzed Water Based Epoxy Semi-Gloss, K46 Series

Eg-Shel Finish

Primer:	ProMar® 200 Zero VOC Interior Latex Primer, B28W2600

1st coat:	ProMar® 200 Zero VOC Interior Latex Eg-Shel, B20-2600 Series

2nd coat:	ProMar® 200 Zero VOC Interior Latex Eg-Shel, B20-2600 Series

Eg-Shel — High Performance Finish

Primer:	ProMar® 200 Zero VOC Interior Latex Primer, B28W2600

1st coat:	Pro Industrial™ Pre-Catalyzed Water Based Epoxy Eg-Shel, K45 Series

2nd coat:	Pro Industrial™ Pre-Catalyzed Water Based Epoxy Eg-Shel, K45 Series

Gypsum Board — Ceiling

Flat Finish

Primer:	ProMar® 200 Zero VOC Interior Latex Primer, B28W2600

1st coat:	ProMar® 400 Zero VOC Interior Latex Flat, B30-4600 Series

2nd coat:	ProMar® 400 Zero VOC Interior Latex Flat, B30-4600 Series

Ferrous Metal — Doors, Frames, and Miscellaneous Metals

Semi-Gloss Finish

Primer:	Pro Industrial™ Pro-Cryl® Universal Primer, B66-310 Series

1st coat:	Pro Industrial™ Acrylic Semi-Gloss, B66-650 Series

2nd coat:	Pro Industrial™ Acrylic Semi-Gloss, B66-650 Series

Ferrous Metal — High Performance Finish for Metal (Including Handrails)

Eg-Shel or Gloss Finish

Primer:	Pro Industrial™ Pro-Cryl®Universal Primer, B66-310 Series

1st coat:	Pro Industrial™ Water Based Catalyzed Epoxy Eg-Shel, B73-360 Series or Gloss, B73-300 Series

2nd coat:	Pro Industrial™ Water Based Catalyzed Epoxy Eg-Shel, B73-360 Series or Gloss, B73-300 Series

Non-Ferrous Metal — Galvanized/Aluminum Surfaces

Semi-Gloss Finish

Primer:	Pro Industrial™ Pro-Cryl® Universal Primer, B66-310 Series

1st coat:	Pro Industrial™ Acrylic Semi-Gloss, B66-650 Series

2nd coat:	Pro Industrial™ Acrylic Semi-Gloss, B66-650 Series

Non-Ferrous Metal — High Performance Finish for Metal (Including Handrails)

Eg-Shel or Gloss Finish

Primer:	Pro Industrial™ Pro-Cryl® Universal Primer, B66-310 Series

1st coat:	Pro Industrial™ Water Based Catalyzed Epoxy Eg-Shel, B73-360 Series or Gloss, B73-300 Series

2nd coat:	Pro Industrial™ Water Based Catalyzed Epoxy Eg-Shel, B73-360 Series or Gloss, B73-300 Series

Conference/Multi-Purpose Room:

Gypsum Board — Walls

Semi-Gloss Finish

Primer:	ProMar® 200 Zero VOC Interior Latex Primer, B28W2600

1st coat:	ProMar® 200 Zero VOC Interior Latex Semi-Gloss, B31-2600 Series

2nd coat:	ProMar® 200 Zero VOC Interior Latex Semi-Gloss, B31-2600 Series

Semi-Gloss — High Performance Finish

Primer:	ProMar® 200 Zero VOC Interior Latex Primer, B28W2600

1st coat:	Pro Industrial™ Pre-Catalyzed Waterbased Epoxy Semi-Gloss, K46 Series

2nd coat:	Pro Industrial™ Pre-Catalyzed Waterbased Epoxy Semi-Gloss, K46 Series

Eg-Shel Finish

Primer:	ProMar® 200 Zero VOC Interior Latex Primer, B28W2600

1st coat:	ProMar® 200 Zero VOC Interior Latex Eg-Shel, B20-2600 Series

2nd coat:	ProMar® 200 Zero VOC Interior Latex Eg-Shel, B20-2600 Series

Eg-Shel — High Performance Finish

Primer:	ProMar® 200 Zero VOC Interior Latex Primer, B28W2600

1st coat:	Pro Industrial™ Pre-Catalyzed Waterbased Epoxy Eg-Shel, K45 Series

2nd coat:	Pro Industrial™ Pre-Catalyzed Waterbased Epoxy Eg-Shel, K45 Series

Gypsum Board — Ceiling

Flat Finish

Primer:	ProMar® 200 Zero VOC Interior Latex Primer, B28W2600

1st coat:	ProMar® 400 Zero VOC Interior Latex Flat, B30-4600 Series

2nd coat:	ProMar® 400 Zero VOC Interior Latex Flat, B30-4600 Series

Ferrous Metal — Doors, Frames, and Miscellaneous Metals

Semi-Gloss Finish

Primer:	Pro Industrial™ Pro-Cryl® Universal Primer, B66-310 Series

1st coat:	Pro Industrial™ Acrylic Semi-Gloss, B66-650 Series

2nd coat:	Pro Industrial™ Acrylic Semi-Gloss, B66-650 Series

Bathroom:

Gypsum Board — Walls

Semi-Gloss Finish

Primer:	ProMar® 200 Zero VOC Interior Latex Primer, B28W2600

1st coat:	ProMar® 200 Zero VOC Interior Latex Semi-Gloss, B31-2600 Series

2nd coat:	ProMar® 200 Zero VOC Interior Latex Semi-Gloss, B31-2600 Series

Semi-Gloss — High Performance Finish

Primer:	ProMar® 200 Zero VOC Interior Latex Primer, B28W2600

1st coat:	Pro Industrial™ Pre-Catalyzed Water Based Epoxy Semi-Gloss, K46 Series

2nd coat:	Pro Industrial™ Pre-Catalyzed Water Based Epoxy Semi-Gloss, K46 Series

Eg-Shel Finish

Primer:	ProMar® 200 Zero VOC Interior Latex Primer, B28W2600

1st coat:	ProMar® 200 Zero VOC Interior Latex Eg-Shel, B20-2600 Series

2nd coat:	ProMar® 200 Zero VOC Interior Latex Eg-Shel, B20-2600 Series

Page 8 —

Bathroom (Cont.):

Eg-Shel — High Performance Finish

Primer:	ProMar® 200 Zero VOC Interior Latex Primer, B28W2600

1st coat:	Pro Industrial™ Pre-Catalyzed Water Based Epoxy Eg-Shel, K45 Series

2nd coat:	Pro Industrial™ Pre-Catalyzed Water Based Epoxy Eg-ShelK45 Series

Gypsum Board — Ceiling

Flat Finish

Primer:	ProMar® 200 Zero VOC Interior Latex Primer, B28W2600

1st coat:	ProMar® 400 Zero VOC Interior Latex Flat, B30-4600 Series

2nd coat:	ProMar® 400 Zero VOC Interior Latex Flat, B30-4600 Series

Ferrous Metal — Doors, Frames, and Miscellaneous Metals

Semi-Gloss Finish

Primer:	Pro Industrial™ Pro-Cryl® Universal Primer, B66-310 Series

1st coat:	Pro Industrial™ Acrylic Semi-Gloss, B66-650 Series

2nd coat:	Pro Industrial™ Acrylic Semi-Gloss, B66-650 Series

Fitness Center:

Gypsum Board — Walls

Semi-Gloss Finish

Primer:	ProMar® 200 Zero VOC Interior Latex Primer, B28W2600

1st coat:	ProMar® 200 Zero VOC Interior Latex Semi-Gloss, B31-2600 Series

2nd coat:	ProMar® 200 Zero VOC Interior Latex Semi-Gloss, B31-2600 Series

Semi-Gloss — High Performance Finish

Primer:	ProMar® 200 Zero VOC Interior Latex Primer, B28W2600

1st coat:	Pro Industrial™ Pre-Catalyzed Water Based Epoxy Semi-Gloss, K46 Series

2nd coat:	Pro Industrial™ Pre-Catalyzed Water Based Epoxy Semi-Gloss, K46 Series

Eg-Shel Finish

Primer:	ProMar® 200 Zero VOC Interior Latex Primer, B28W2600

1st coat:	ProMar® 200 Zero VOC Interior Latex Eg-Shel, B20-2600 Series

2nd coat:	ProMar® 200 Zero VOC Interior Latex Eg-Shel, B20-2600 Series

Eg-Shel — High Performance Finish

Primer:	ProMar® 200 Zero VOC Interior Latex Primer, B28W2600

1st coat:	Pro Industrial™ Pre-Catalyzed Water Based Epoxy Eg-Shel, K45 Series

2nd coat:	Pro Industrial™ Pre-Catalyzed Water Based Epoxy Eg-Shel, K45 Series

Gypsum Board — Ceiling

Flat Finish

Primer:	ProMar® 200 Zero VOC Interior Latex Primer, B28W2600

1st coat:	ProMar® 400 Zero VOC Interior Latex Flat, B30-4600 Series

2nd coat:	ProMar® 400 Zero VOC Interior Latex Flat, B30-4600 Series

Ferrous Metal — Doors, Frames, and Miscellaneous Metals

Semi-Gloss Finish

Primer:	Pro Industrial™ Pro-Cryl® Universal Primer, B66-310 Series

1st coat:	Pro Industrial™ Acrylic Semi-Gloss, B66-650 Series

2nd coat:	Pro Industrial™ Acrylic Semi-Gloss, B66-650 Series

— Page 9

— Page 10

Page 10 —

Restaurant/Kitchen:

Gypsum Board

Semi-Gloss Finish

Primer:	ProMar® 200 Zero VOC Interior Latex Primer, B28W2600

1st coat:	Pro Industrial™ Pre-Catalyzed Water Based Epoxy Semi-Gloss, K46 Series

2nd coat:	Pro Industrial™ Pre-Catalyzed Water Based Epoxy Semi-Gloss, K46 Series

Eg-Shel Finish

Primer:	ProMar® 200 Zero VOC Interior Latex Primer, B28W2600

1st coat:	Pro Industrial™ Pre-Catalyzed Water Based Epoxy Eg-Shel, K45 Series

2nd coat:	Pro Industrial™ Pre-Catalyzed Water Based Epoxy Eg-Shel, K45 Series

Eg-Shel or Gloss — High Performance Finish

Primer:	ProMar® 200 Zero VOC Interior Latex Primer, B28W2600

1st coat:	Pro Industrial™ Water Based Catalyzed Epoxy Eg-Shel, B73-360 Series or Gloss, B73-300 Series

2nd coat:	Pro Industrial™ Water Based Catalyzed Epoxy Eg-Shel, B73-360 Series or Gloss, B73-300 Series

Gypsum Board — Ceiling

Flat Finish

Primer:	ProMar® 200 Zero VOC Interior Latex Primer, B28W2600

1st coat:	ProMar® 400 Zero VOC Interior Latex Flat, B30-4600 Series

2nd coat:	ProMar® 400 Zero VOC Interior Latex Flat, B30-4600 Series

Ferrous Metal — High Performance Finish (Including Handrails)

Eg-Shel or Gloss Finish

Primer:	Pro Industrial™ Pro-Cryl® Universal Primer, B66-310 Series

1st coat:	Pro Industrial™ Water Based Catalyzed Epoxy Eg-Shel, B73-360 Series or Gloss, B73-300 Series

2nd coat:	Pro Industrial™ Water Based Catalyzed Epoxy Eg-Shel, B73-360 Series or Gloss, B73-300 Series

Non-Ferrous Metal — High Performance Finish (Including Handrails)

Eg-Shel or Gloss Finish

Primer:	Pro Industrial™ Pro-Cryl® Universal Primer, B66-310 Series

1st coat:	Pro Industrial™ Water Based Catalyzed Epoxy Eg-Shel, B73-360 Series or Gloss, B73-300 Series

2nd coat:	Pro Industrial™ Water Based Catalyzed Epoxy Eg-Shel, B73-360 Series or Gloss, B73-300 Series

Non-Ferrous Metal — Galvanized/Aluminum Surfaces

Semi-Gloss Finish

Primer:	Pro Industrial™ Pro-Cryl® Universal Primer, B66-310 Series

1st coat:	Pro Industrial™ Acrylic Semi-Gloss, B66-650 Series

2nd coat:	Pro Industrial™ Acrylic Semi-Gloss, B66-650 Series

Bar:

Gypsum Board

Semi-Gloss Finish

Primer:	ProMar® 200 Zero VOC Interior Latex Primer, B28W2600

1st coat:	Pro Industrial™ Pre-Catalyzed Water Based Epoxy Semi-Gloss, K46 Series

2nd coat:	Pro Industrial™ Pre-Catalyzed Water Based Epoxy Semi-Gloss, K46 Series

Eg-Shel Finish

Primer:	ProMar® 200 Zero VOC Interior Latex Primer, B28W2600

1st coat:	Pro Industrial™ Pre-Catalyzed Water Based Epoxy Eg-Shel, K45 Series

2nd coat:	Pro Industrial™ Pre-Catalyzed Water Based Epoxy Eg-Shel, K45 Series

Eg-Shel or Gloss — High Performance Finish

Primer:	ProMar® 200 Zero VOC Interior Latex Primer, B28W2600

1st coat:	Pro Industrial™ Water Based Catalyzed Epoxy Eg-Shel, B73-360 Series or Gloss, B73-300 Series

2nd coat:	Pro Industrial™ Water Based Catalyzed Epoxy Eg-Shel, B73-360 Series or Gloss, B73-300 Series

Gypsum Board — Ceiling

Flat Finish

Primer:	ProMar® 200 Zero VOC Interior Latex Primer, B28W2600

1st coat:	ProMar® 400 Zero VOC Interior Latex Flat, B30-4600 Series

2nd coat:	ProMar® 400 Zero VOC Interior Latex Flat, B30-4600 Series

Ferrous Metal — High Performance Finish (Including Handrails)

Eg-Shel or Gloss Finish

Primer:	Pro Industrial™ Pro-Cryl® Universal Primer, B66-310 Series

1st coat:	Pro Industrial™ Water Based Catalyzed Epoxy Eg-Shel, B73-360 Series or Gloss, B73-300 Series

2nd coat:	Pro Industrial™ Water Based Catalyzed Epoxy Eg-Shel, B73-360 Series or Gloss, B73-300 Series

Non-Ferrous Metal — Galvanized/Aluminum Surfaces

Semi-Gloss Finish

Primer:	Pro Industrial™ Pro-Cryl® Universal Primer, B66-310 Series

1st coat:	Pro Industrial™ Acrylic Semi-Gloss, B66-650 Series

2nd coat:	Pro Industrial™ Acrylic Semi-Gloss, B66-650 Series

Non-Ferrous Metal — High Performance Finish (Including Handrails)

Eg-Shel or Gloss Finish

Primer:	Pro Industrial™ Pro-Cryl® Universal Primer, B66-310 Series

1st coat:	Pro Industrial™ Water Based Catalyzed Epoxy Eg-Shel, B73-360 Series or Gloss, B73-300 Series

2nd coat:	Pro Industrial™ Water Based Catalyzed Epoxy Eg-Shel, B73-360 Series or Gloss, B73-300 Series

— Page 11

Page 12 —

Nightclub:

Gypsum Board — Walls

Semi-Gloss Finish

Primer:	ProMar® 200 Zero VOC Interior Latex Primer, B28W2600

1st coat:	ProMar® 200 Zero VOC Interior Latex Semi-Gloss, B31-2600 Series

2nd coat:	ProMar® 200 Zero VOC Interior Latex Semi-Gloss, B31-2600 Series

Semi-Gloss — High Performance Finish

Primer:	ProMar® 200 Zero VOC Interior Latex Primer, B28W2600

1st coat:	Pro Industrial™ Pre-Catalyzed Water Based Epoxy Semi-Gloss, K46 Series

2nd coat:	Pro Industrial™ Pre-Catalyzed Water Based Epoxy Semi-Gloss, K46 Series

Eg-Shel Finish

Primer:	ProMar® 200 Zero VOC Interior Latex Primer, B28W2600

1st coat:	ProMar® 200 Zero VOC Interior Latex Eg-Shel, B20-2600 Series

2nd coat:	ProMar® 200 Zero VOC Interior Latex Eg-Shel, B20-2600 Series

Eg-Shel — High Performance Finish

Primer:	ProMar® 200 Zero VOC Interior Latex Primer, B28W2600

1st coat:	Pro Industrial™ Pre-Catalyzed Water Based Epoxy Eg-Shel, K45 Series

2nd coat:	Pro Industrial™ Pre-Catalyzed Water Based Epoxy Eg-Shel, K45 Series

Gypsum Board — Ceiling

Flat Finish

Primer:	ProMar® 200 Zero VOC Interior Latex Primer, B28W2600

1st coat:	ProMar® 400 Zero VOC Interior Latex Flat, B30-4600 Series

2nd coat:	ProMar® 400 Zero VOC Interior Latex Flat, B30-4600 Series

Ferrous Metal — Doors, Frames, and Miscellaneous Metals

Semi-Gloss Finish

Primer:	Pro Industrial™ Pro-Cryl® Universal Primer, B66-310 Series

1st coat:	Pro Industrial™ Acrylic Semi-Gloss, B66-650 Series

2nd coat:	Pro Industrial™ Acrylic Semi-Gloss, B66-650 Series

Ferrous Metal — High Performance Finish (Including Handrails)

Eg-Shel or Gloss Finish

Primer:	Pro Industrial™ Pro-Cryl® Universal Primer, B66-310 Series

1st coat:	Pro Industrial™ Water Based Catalyzed Epoxy Eg-Shel, B73-360 Series or Gloss, B73-300 Series

2nd coat:	Pro Industrial™ Water Based Catalyzed Epoxy Eg-Shel, B73-360 Series or Gloss, B73-300 Series

— Page 13

Casino/Gaming:

Gypsum Board — Walls

Semi-Gloss Finish

Primer:	ProMar® 200 Zero VOC Interior Latex Primer, B28W2600

1st coat:	ProMar® 200 Zero VOC Interior Latex Semi-Gloss, B31-2600 Series

2nd coat:	ProMar® 200 Zero VOC Interior Latex Semi-Gloss, B31-2600 Series

Semi-Gloss — High Performance Finish

Primer:	ProMar® 200 Zero VOC Interior Latex Primer, B28W2600

1st coat:	Pro Industrial™ Pre-Catalyzed Water Based Epoxy Semi-Gloss, K46 Series

2nd coat:	Pro Industrial™ Pre-Catalyzed Water Based Epoxy Semi-Gloss, K46 Series

Eg-Shel Finish

Primer:	ProMar® 200 Zero VOC Interior Latex Primer, B28W2600

1st coat:	ProMar® 200 Zero VOC Interior Latex Eg-Shel, B20-2600 Series

2nd coat:	ProMar® 200 Zero VOC Interior Latex Eg-Shel, B20-2600 Series

Eg-Shel — High Performance Finish

Primer:	ProMar® 200 Zero VOC Interior Latex Primer, B28W2600

1st coat:	Pro Industrial™ Pre-Catalyzed Water Based Epoxy Eg-Shel, K45 Series

2nd coat:	Pro Industrial™ Pre-Catalyzed Water Based Epoxy Eg-Shel, K45 Series

Gypsum Board — Ceiling

Flat Finish

Primer:	ProMar® 200 Zero VOC Interior Latex Primer, B28W2600

1st coat:	ProMar® 400 Zero VOC Interior Latex Flat, B30-4600 Series

2nd coat:	ProMar® 400 Zero VOC Interior Latex Flat, B30-4600 Series

Ferrous Metal — Doors, Frames, and Miscellaneous Metals

Semi-Gloss Finish

Primer:	Pro Industrial™ Pro-Cryl® Universal Primer, B66-310 Series

1st coat:	Pro Industrial™ Acrylic Semi-Gloss, B66-650 Series

2nd coat:	Pro Industrial™ Acrylic Semi-Gloss, B66-650 Series

Ferrous Metal — High Performance Finish (Including Handrails)

Eg-Shel or Gloss Finish

Primer:	Pro Industrial™ Pro-Cryl® Universal Primer, B66-310 Series

1st coat:	Pro Industrial™ Water Based Catalyzed Epoxy  Eg-Shel, B73-360 Series or Gloss, B73-300 Series

2nd coat:	Pro Industrial™ Water Based Catalyzed Epoxy Eg-Shel, B73-360 Series or Gloss, B73-300 Series

Non-Ferrous Metal — Galvanized/Aluminum Surfaces

Semi-Gloss Finish

Primer:	Pro Industrial™ Pro-Cryl® Universal Primer, B66-310 Series

1st coat:	Pro Industrial™ Acrylic Semi-Gloss, B66-650 Series

2nd coat:	Pro Industrial™ Acrylic Semi-Gloss, B66-650 Series

Non-Ferrous Metal — High Performance Finish (Including Handrails)

Eg-Shel or Gloss Finish

Primer:	Pro Industrial™ Pro-Cryl® Universal Primer, B66-310 Series

1st coat:	Pro Industrial™ Water Based Catalyzed Epoxy Eg-Shel, B73-360 Series or Gloss, B73-300 Series

2nd coat:	Pro Industrial™ Water Based Catalyzed Epoxy Eg-Shel, B73-360 Series or Gloss, B73-300 Series

Healthcare Facility Exterior Paint Schedule

Specifier Notes — Topcoats:

a. A-100 Exterior Latex — A 100% acrylic formula that offers a durable, professional quality coating available in a Flat,

Satin, and Gloss line of finishes.

b. Loxon XP — A high build exterior masonry coating that can be applied to fresh concrete and masonry surfaces, saving

you time and labor and accelerating construction schedules. It is highly alkali and efflorescent resistant.

c. Conflex XL Elastomeric — An elastomeric coating that provides excellent flexibility, durability, and weather resistance.

This product will protect against wind-driven rain when used on tilt-up, precast, or poured-in-place concrete, CMU, and

stucco. Available in smooth or textured finishes.

d. Waterbased Acrolon 100 Urethane — A two-component, less than 100 g/L VOC, waterbased, acrylic urethane

enamel. It is designed for high performance protection with outstanding exterior gloss and color retention properties.

e. Pro Industrial Zero VOC Acrylic — A single-component, light industrial, 100% acrylic coating that flows and levels to a

silky smooth finish. Available in a Satin, Semi-Gloss, and Gloss line of finishes.

f. UltraCrete Textured Masonry Topcoat — A 100% acrylic aggregate filled coating used to produce a textured finish

on exterior surfaces

BOH Areas (Mechanical/Electrical Rooms, Shop Areas, Distribution Areas):

Gypsum Board — Walls

Semi-Gloss Finish

Primer:	ProMar® 200 Zero VOC Interior Latex Primer, B28W2600

1st coat:	ProMar® 200 Zero VOC Interior Latex Semi-Gloss, B31-2600 Series

2nd coat:	ProMar® 200 Zero VOC Interior Latex Semi-Gloss, B31-2600 Series

Semi-Gloss — High Performance Finish

Primer:	ProMar® 200 Zero VOC Interior Latex Primer, B28W2600

1st coat:	Pro Industrial™ Pre-Catalyzed Water Based Epoxy Semi-Gloss, K46 Series

2nd coat:	Pro Industrial™ Pre-Catalyzed Water Based Epoxy Semi-Gloss, K46 Series

Eg-Shel Finish

Primer:	ProMar® 200 Zero VOC Interior Latex Primer, B28W2600

1st coat:	ProMar® 200 Zero VOC Interior Latex Eg-Shel, B20-2600 Series

2nd coat:	ProMar® 200 Zero VOC Interior Latex Eg-Shel, B20-2600 Series

Eg-Shel — High Performance Finish

Primer:	ProMar® 200 Zero VOC Interior Latex Primer, B28W2600

1st coat:	Pro Industrial™ Pre-Catalyzed Water Based Epoxy Eg-Shel, K45 Series

2nd coat:	Pro Industrial™ Pre-Catalyzed Water Based Epoxy Eg-Shel, K45 Series

Gypsum Board — Ceiling

Flat Finish

Primer:	ProMar® 200 Zero VOC Interior Latex Primer, B28W2600

1st coat:	ProMar® 400 Zero VOC Interior Latex Flat, B30-4600 Series

2nd coat:	ProMar® 400 Zero VOC Interior Latex Flat, B30-4600 Series

Warehouse/Distribution

CMU — Concrete Masonry Units

Eg-Shel Finish

Primer: 	Loxon® Block Surfacer, A24W200

1st coat: 	ProMar® 200 Zero VOC Interior Latex Eg-Shel, B20-2600 Series

2nd coat: 	ProMar® 200 Zero VOC Interior Latex Eg-Shel, B20-2600 Series

Masonry and Concrete — Walls

Eg-Shel Finish

Primer: 	Loxon® Concrete & Masonry Primer Sealer, A24W8300

1st coat: 	ProMar® 200 Zero VOC Interior Latex Eg-Shel, B20-2600 Series

2nd coat: 	ProMar® 200 Zero VOC Interior Latex Eg-Shel, B20-2600 Series

Ferrous Decking — Including Bar Joists

Flat, Eg-Shel, or Semi-Gloss Finish

Primer:  	Pro Industrial™ Pro-Cryl® Universal Primer, B66-310 Series

1st coat:  	Pro Industrial™ Waterborne Acrylic Dryfall, Flat, Eg-Shel, Semi-Gloss, B42-80 Series

2nd coat:  	Pro Industrial™ Waterborne Acrylic Dryfall, Flat, Eg-Shel, Semi-Gloss, B42-80 Series

Non-Ferrous Metal Decking

Flat, Eg-Shel, or Semi-Gloss Finish

Primer:  	Pro Industrial™ Pro-Cryl® Universal Primer, B66-310 Series

1st coat:  	Pro Industrial™ Waterborne Acrylic Dryfall, Flat, Eg-Shel, Semi-Gloss, B42-80 Series

2nd coat:  	Pro Industrial™ Waterborne Acrylic Dryfall, Flat, Eg-Shel, Semi-Gloss, B42-80 Series

Page 14 —

— Page 15

BOH Areas (Mechanical/Electrical Rooms, Shop Areas, and Distribution Areas) (Cont.):

Ferrous Metal — Doors, Frames, and Miscellaneous Metals

Semi-Gloss Finish

Primer:  	Pro Industrial™ Pro-Cryl® Universal Primer, B66-310 Series

1st coat:  	Pro Industrial™ Acrylic Semi-Gloss, B66-650 Series

2nd coat:  	Pro Industrial™ Acrylic Semi-Gloss, B66-650 Series

Ferrous Metal — High Performance Finish (Including Handrails)

Eg-Shel or Gloss Finish

Primer:  	Pro Industrial™ Pro-Cryl® Universal Primer, B66-310 Series

1st coat:  	Pro Industrial™ Water Based Catalyzed Epoxy Eg-Shel, B73-360 Series or Gloss, B73-300 Series

2nd coat:   	Pro Industrial™ Water Based Catalyzed Epoxy Eg-Shel, B73-360 Series or Gloss, B73-300 Series

Boiler Room/Physical Plant/Maintenance Office:

Masonry and Concrete Walls and Ceilings

Eg-Shel Finish

Primer:  	Loxon® Concrete & Masonry Primer Sealer, A24W8300

1st coat:  	Pro Industrial™ Pre-Catalyzed Water Based Epoxy Eg-Shel, K45 Series

2nd coat:  	Pro Industrial™ Pre-Catalyzed Water Based Epoxy Eg-Shel, K45 Series

Masonry and Concrete Ceilings

Flat Finish

Primer:	Loxon® Concrete & Masonry Primer Sealer, A24W8300

1st coat:	ProMar® 400 Zero VOC Interior Latex Flat, B30-4600 Series

2nd coat:	ProMar® 400 Zero VOC Interior Latex Flat, B30-4600 Series

Ferrous Metal — Doors, Frames, and Miscellaneous Metals

Semi-Gloss Finish

Primer:  	Pro Industrial™ Pro-Cryl® Universal Primer, B66-310 Series

1st coat:  	Pro Industrial™ Acrylic Semi-Gloss, B66-650 Series

2nd coat:  	Pro Industrial™ Acrylic Semi-Gloss, B66-650 Series

Ferrous Metal — High Performance Finish (Including Handrails)

Eg-Shel or Gloss Finish

Primer:  	Pro Industrial™ Pro-Cryl® Universal Primer, B66-310 Series

1st coat:  	Pro Industrial™ Water Based Catalyzed Epoxy Eg-Shel, B73-360 Series or Gloss, B73-300 Series

2nd coat:  	Pro Industrial™ Water Based Catalyzed Epoxy Eg-Shel, B73-360 Series or Gloss, B73-300 Series

Page 16 —

Indoor/Outdoor Pool:

Pool Deck — Waterproofing Deck Coating

Please contact your local Sherwin-Williams Sales Representative to evaluate the project for the most complete product recommendation.

CMU — Interior Walls in Pool Area & Chemical Storage

Eg-Shel or Gloss Finish

Filler coat:	Heavy Duty Block Filler B42W46

1st coat:	Pro Industrial™ Water Based Catalyzed Epoxy Eg-Shel, B73-360 Series or Gloss, B73-300 Series

2nd coat:	Pro Industrial™ Water Based Catalyzed Epoxy Eg-Shel, B73-360 Series or Gloss, B73-300 Series

Gypsum Board — High Performance Finish for Walls & Ceilings

Gloss Finish

Primer:	ProMar® 200 Zero VOC Interior Latex Primer, B28W2600

1st coat:	Pro Industrial™ Water Based Catalyzed Epoxy Eg-Shel, B73-360 Series or Gloss, B73-300 Series

2nd coat:	Pro Industrial™ Water Based Catalyzed Epoxy Eg-Shel, B73-360 Series or Gloss, B73-300 Series

Faux Impressions — Decorative Wall Finishes; Metallic and

High-Polish Fresco (Venetian Plaster):

Faux Impressions Metallic Finish

Primer:	ProMar® 200 Zero VOC Interior Latex Primer, B28W2600

1st coat:	ProMar® 200 Zero VOC Interior Latex Eg-Shel B20-2600 Series

2nd coat:	ProMar® 200 Zero VOC Interior Latex Eg-Shel B20-2600 Series

3rd coat:	Faux Impressions® Metallic Glaze

Faux Impressions Metallic Finish

Primer:	ProMar® 200 Zero VOC Interior Latex Primer, B28W2600

1st coat:       ProMar® 200 Zero VOC Interior Latex Eg-Shel B20-2600 Series

2nd coat:      ProMar® 200 Zero VOC Interior Latex Eg-Shel, B20-2600 Series

3rd coat:	Faux Impressions® Metallic Glaze

4th coat:	Sher-Clear™ Acrylic Clear Coat High Gloss, B66C375

Faux Impressions High Polish Fresco (Venetian Plaster)

Walls, Gypsum Board (Level 5 Drywall Finish required)

1st coat:	ProMar® 200 Zero VOC Interior Latex Primer, B28W2600

2nd coat:	Faux Impressions® Venetian Plaster

3rd coat:	Faux Impressions® Venetian Plaster

4th coat:	Faux Impressions® Venetian Plaster

— Page 17

Concrete Floors — Pigmented Coating Systems:

Light Duty (Foot Traffic)

1st coat:  		ArmorSeal® Tread-Plex™

2nd coat:  	ArmorSeal® Tread-Plex™

Medium/Heavy Duty (Kitchen Areas, Food Service, Restrooms, Labs, Corridors)

Gloss Finish

1st coat: 	 ArmorSeal® 8100 Finish B70-8100 Series

2nd coat: 	 ArmorSeal® 8100 Finish B70-8100 Series

Satin Finish

1st coat: 	 ArmorSeal® 8100 Finish B70-8160 Series

2nd coat: 	 ArmorSeal® 8100 Finish B70-8160 Series

Concrete Floors — Decorative Coating Systems:

Sherwin-Williams, offers a full line of patching materials, basecoats and high-performance floor finishes. 
Sherwin-Williams has a coating solution for most surfaces.

Please contact your local Sherwin-Williams representative for detailed information and specifications.

Casino Facility Exterior Paint Schedule

Specifier Notes — Topcoats:

a. A-100® Exterior Latex — A 100% acrylic formula that offers a durable, professional quality coating available in a

Flat, Satin, Low-Sheen, and Gloss finishes.

b. Loxon XP™ Masonry Coating — Provides flexibility, durability, and weather resistance. This product will protect against wind-driven rain when used on tilt-up, precast, or poured-in-place concrete, CMU, and stucco. This may be applied to a surface with a pH of 6 to 13.

c. Water Based Acrolon™ 100 Urethane — A two-component, water based, acrylic urethane enamel. It is designed for high performance protection with outstanding exterior gloss and color retention properties.

d. Pro Industrial™ Acrylic Coating — A single-component, light industrial, 100% acrylic coating that flows and levels to a smooth finish. Available in an Eg-Shel, Semi-Gloss, and Gloss finishes.

e. UltraCrete Textured Masonry Topcoat — A 100% acrylic aggregate filled coating used to produce a textured

finish on exterior surfaces.

Recommended Finishes:

Body:		Flat or Satin (unless otherwise noted)

Trim:		Semi-Gloss or Gloss Finish (unless otherwise noted)

Page 18 —

Concrete and Masonry Vertical Surfaces — Smooth:

Gloss Finish

Primer:	Loxon® Concrete & Masonry Primer Sealer, A24W8300

1st coat:	A-100® Exterior Latex Gloss, A8 Series

2nd coat:	A-100® Exterior Latex Gloss, A8 Series

Satin Finish

Primer:	Loxon® Concrete & Masonry Primer Sealer, A24W8300

1st coat:	A-100® Exterior Latex Satin, A82 Series

2nd coat:	A-100® Exterior Latex Satin, A82 Series

Low-Sheen Finish

Primer:	Loxon® Concrete & Masonry Primer Sealer, A24W8300

1st coat:	A-100® Exterior Latex Low Sheen, A12 Series

2nd coat:	A-100® Exterior Latex Low Sheen, A12 Series

Flat Finish

Primer:	Loxon® Concrete & Masonry Primer Sealer, A24W8300

1st coat:	A-100® Exterior Latex Flat, A6 Series

2nd coat:	A-100® Exterior Latex Flat, A6 Series

Flat Finish — High Build Coating

1st coat:	Loxon XP™ Masonry Coating, A24-1400 Series

2nd coat:	Loxon XP™ Masonry Coating, A24-1400 Series

Textured Finish

Primer:	Loxon® Concrete & Masonry Primer Sealer, A24W8300

1st coat:	Ultracrete Textured Masonry Topcoat Fine, Medium, Extra Coarse, A44-800 Series

2nd coat:	Ultracrete Textured Masonry Topcoat Fine, Medium, Extra Coarse, A44-800 Series

Plaster/Stucco/EIFS:

Gloss Finish

Primer:	Loxon® Concrete & Masonry Primer Sealer, A24W8300

1st coat:	A-100® Exterior Latex Gloss, A8 Series

2nd coat:	A-100® Exterior Latex Gloss, A8 Series

Satin Finish

Primer:	Loxon® Concrete & Masonry Primer Sealer, A24W8300

1st coat:	A-100® Exterior Latex Satin, A82 Series

2nd coat:	A-100® Exterior Latex Satin, A82 Series

Low-Sheen Finish

Primer:	Loxon® Concrete & Masonry Primer Sealer, A24W8300

1st coat:	A-100® Exterior Latex Low Sheen, A12 Series

2nd coat:	A-100® Exterior Latex Low Sheen, A12 Series

Flat Finish

Primer:	Loxon® Concrete & Masonry Primer Sealer, A24W8300

1st coat:	A-100® Exterior Latex Flat, A6 Series

2nd coat:	A-100® Exterior Latex Flat, A6 Series

Flat Finish — High Build Coating

1st coat:	Loxon XP™ Masonry Coating, A24-1400 Series

2nd coat:	Loxon XP™ Masonry Coating, A24-1400 Series

— Page 19

Wood:

Satin Finish

Primer:	Exterior Latex Wood Primer, B42W8041

1st coat:	A-100® Exterior Latex Satin, A82 Series

2nd coat:	A-100® Exterior Latex Satin, A82 Series

Low-Sheen Finish

Primer:	Exterior Latex Wood Primer, B42W8041

1st coat:	A-100® Exterior Latex Low Sheen, A12 Series

2nd coat:	A-100® Exterior Latex Low Sheen, A12 Series

Ferrous Metals:

Gloss — High Performance Finish (Areas where performance is critical such as railings, canopies, handrails, etc.)

Primer:	Pro Industrial™ Pro-Cryl® Universal Primer, B66-310 Series

1st coat:  	Water Based Acrolon™ 100 Polyurethane Gloss, B65-700 Series

2nd coat:  	Water Based Acrolon™ 100 Polyurethane Gloss, B65-700 Series

Semi-Gloss Finish

Primer:	Pro Industrial™ Pro-Cryl® Universal Primer, B66-310 Series

1st coat:	Pro Industrial™ Acrylic Semi-Gloss, B66-650 Series

2nd coat:	Pro Industrial™ Acrylic Semi-Gloss, B66-650 Series

Non-Ferrous Metals:

Gloss — High Performance Finish (Areas where performance is critical such as railings, canopies, handrails, etc.)

Primer:	Pro Industrial™ Pro-Cryl® Universal Primer, B66-310 Series

1st coat:  	Water Based Acrolon™ 100 Polyurethane Gloss, B65-720 Series

2nd coat:  	Water Based Acrolon™ 100 Polyurethane Gloss, B65-720 Series

Semi-Gloss Finish

Primer:	Pro Industrial™ Pro-Cryl® Universal Primer, B66-310 Series

1st coat:	Pro Industrial™ Acrylic Semi-Gloss, B66-650 Series

2nd coat:	Pro Industrial™ Acrylic Semi-Gloss, B66-650 Series

Balcony:

Concrete Deck — Balcony Floors

Flat Finish

1st coat:	Sher-Crete® Flexible Concrete Waterproofer, A5 Series

2nd coat:	Sher-Crete® Flexible Concrete Waterproofer, A5 Series

Non-Ferrous Metal — Railing

Gloss Finish

Primer:	Pro Industrial™ Pro-Cryl® Universal Primer, B66-310 Series

1st coat:  	Water Based Acrolon™ 100 Polyurethane Gloss, B65-720 Series

2nd coat:  	Water Based Acrolon™ 100 Polyurethane Gloss, B65-720 Series

Page 20 —

©2015 The Sherwin-Williams Company